Project Overview

You will design and simulate an 8-bit Arithmetic Logic Unit (ALU) using a Hardware Description Language (HDL) of your choice (e.g., VHDL, Verilog). The project is to be completed structurally, emphasizing the hardware components rather than behavioral descriptions.

Team Composition

•	
      Teams of 1–4 students.

Project Requirements

Your ALU must support the following arithmetic operations:

1	
      Addition

2	
      Subtraction

3	
      Multiplication

4	
      Division

Multiplication Algorithms

You must implement one multiplication algorithm of your choice, from the following options studied during the laboratory:

•	
      Booth Radix-2

•	
      Modified Booth Algorithm

•	
      Booth Radix-4

•	
      Booth Radix-8

Division Algorithms

You must implement one division algorithm of your choice, from the following options studied during the laboratory:

•	
      Restoring Division

•	
      Non-restoring Division

•	
      SRT Radix-2

•	
      SRT Radix-4

Structural Implementation

Your HDL code should be structurally described, which means:

•	
      Clearly defined hardware modules and interconnections

•	
      Separate implementation for Arithmetic Unit, Control Unit, Multiplexer, Registers, and other necessary hardware components

•	
      Minimal or no behavioral code

Control Unit

You must design a Control Unit capable of emitting appropriate control signals to select and execute each arithmetic operation.

Recommended Tools

You may use any HDL simulation tool of your choice. Recommended tools include: ModelSim, Quartus Prime, Xilinx Vivado, Icarus Verilog

Project Phases

The project will be divided into three primary phases:

1. Design Phase

•	
      Define the architecture and component modules.

•	
      Provide schematic diagrams detailing module connections and signal flow.

•	
      Clearly indicate the algorithm choices for multiplication and division.

2. Implementation Phase

•	
      Write structurally-oriented HDL code.

•	
      Implement each arithmetic module individually.

•	
      Integrate modules with the Control Unit to form a fully operational 8-bit ALU.

3. Testing Phase

•	
      Develop comprehensive testbenches for each arithmetic operation.

•	
      Perform simulations to verify correctness.

•	
      Provide simulation waveforms demonstrating successful operation.

Deliverables

Your final submission should include:

•	
      Documentation:

◦	
      Project overview and objectives

◦	
      Architecture diagrams and explanations

◦	
      HDL code listings (clearly commented and structured)

◦	
      Testbench descriptions and simulation waveforms

◦	
      Discussion of results, including issues encountered and resolutions

•	
      Presentation:

◦	
      Prepare a concise presentation (~10 minutes) summarizing design decisions, structural implementations, testing procedures, and final results.

◦	
      Presentations will be conducted during Week 9 of the laboratory sessions.

Evaluation Criteria

•	
      Correctness and functionality of arithmetic operations

•	
      Clarity and structural organization of HDL implementation

•	
      Completeness of testing and accuracy of simulations

•	
      Quality and clarity of project documentation and presentation

•	
      Team collaboration and individual contribution (where applicable)